| File or Folder Name | Type | Description |
| --- | --- | --- |
| InterpolatedCoastalVLM | CSV | A four column data set of vertical land movement estimates and their uncertainties for 171 locations along the coast of Washington. Column 1 = Longitude, Column 2 = Latitude, Column 3 = vertical land movement rate in mm/yr and Column 4 = uncertainty. The uncertainty value is the standard deviation of 1000 interpolated surfaces calculated by randomly varying the vertical land movement observations in VLMObservations.xls and the tectonic locking model points in TectonicLockingModelPoints.csv |
| TectonicLockingModelPoints | CSV | A three column data set of vertical bedrock movement rates derived from a tectonic locking model. See Appendix C at http://www.wacoastalnetwork.com/wcrp-documents.html for full details. Column 1 = longitude, Column 2 = latitude, Column 3 = estimated vertical rate in mm/yr. The uncertainty for each of these points was assigned; ±2 mm/yr |
| VLMObservations | XLS | An 8 column XLS worksheet summarizing the vertical land movement observations used in our analysis. Column 1 = station name or identifier, Column 2 = longitude, Column 3 = latitude, Column 4 = vertical land movement rate in mm/yr, Column 5 = the standard deviation of the velocity estimate in mm/yr, Column 6 = the data type (CGPS, leveling or water level), Column 7 = a quality index for the CGPS data, where 1 = high quality and 2=low quality, Column 8 = an index used to identify whether the station is within our “area of interest” for this project. See Appendix C at http://www.wacoastalnetwork.com/wcrp-documents.html for additional details. |

Description of supplementary files for:

Miller, I.M., Morgan, H., Mauger, G., Newton, T., Weldon, R., Schmidt, D., Welch, M., Grossman, E. 2018. Projected Sea Level Rise for Washington State – A 2018 Assessment. A collaboration of Washington Sea Grant, University of Washington Climate Impacts Group, Oregon State University, University of Washington, and US Geological Survey. Prepared for the Washington Coastal Resilience Project.

Originally released 30 July 2018.